CHIP DESIGN ASSOCIATE  ·  O-LEVEL ‘CHIP DESIGN’

Module 2 — Topic 2

RTL Design Methodology

Tutorial & Practice Workbook

A self-study companion to the Topic 2 slide deck. It explains every concept the deck introduces and why it exists, walks you through nine guided tutorials at the keyboard, and ends with 103 graded exercises and full worked solutions. Every number quoted here was produced by running the code in Topic2_Lab/. Nothing in this workbook requires you to look anything up elsewhere.

NOS: NIE/ELE/N0102  ·  Module 2 “Verilog RTL coding for Synthesis”, subtopic 2 “RTL Design Methodology”  ·  Syllabus: basics of register transfer level (RTL) design; overview of RTL design process and methodology; introduction to hardware description languages (HDLs) such as Verilog or VHDL.  Module duration: 25 h theory, 35 h practical, of which RTL Design and Implementation Labs are 40 h.

# Terminal Outcomes

After successful completion of Module 2, the student shall be able to:

Module 2 terminal outcomes (NOS NIE/ELE/N0102) 1. Understand the design cycle of VLSI. 2. Understand Verilog programming syntax, level of abstraction in Verilog programming, and testbench simulation. 3. Design and develop IPs for VLSI using Verilog. 4. Emulate, debug and characterise reusable IPs.

Outcome 2 names this subtopic in the NOS itself. "Level of abstraction" is not a phrase this workbook invented to organise the material — it is the deliverable. Part 1 takes one circuit down all four levels of abstraction Verilog offers, simulates them together, and proves they are the same circuit.

Outcome 3 begins with knowing what is synthesisable and what is not, which is Part 2. Outcome 4 asks for reusable IP, and reuse is a set of decisions you take while writing, not a property you add afterwards — also Part 2.

# Key Learning Outcomes

| Theory — you will be able to | Practical — you will be able to |
| --- | --- |
| Explain what register transfer level means, and the timing model it implies | Write RTL code for basic digital circuits |
| Describe the levels of abstraction available in an HDL, and choose between them with a reason | Validate RTL designs through simulation using testbenches |
| Explain how the synthesisable subset and the coding rules shape the RTL you are allowed to write | Apply a coding standard and check it mechanically rather than by review |
| Compare Verilog and VHDL, and read either | Synthesise a design and read what the tool actually built |

Each outcome is assessed by a command you run make transfer, make ladder → what RTL means, and abstraction make prove, make mux → equivalence, and what it does NOT tell you make subset, make mismatch → the synthesisable subset, and its traps make pitfalls → the latch and the blocking trap, measured make lint, make lintcheck → the coding standard, mechanised make fsm → state machines, both styles, both encodings make dpctrl → datapath and controller make reuse → parameters, hierarchy and generate make langs → Verilog or VHDL, on two designs make flow → the whole methodology, end to end

What's inside THEORY Part 1 What RTL is — the two kinds of logic, the synchronous discipline, registers and transfers, blocking against non-blocking, four levels of abstraction, proof, and the running example Part 2 The design process and methodology — the flow, the synthesisable subset, inferred latches, the simulate/synthesise mismatch, seven coding rules, micro-architecture, reuse and coding style Part 3 The patterns every block is built from — datapath and controller, the finite state machine, Moore against Mealy, state encoding, and parameters, hierarchy and generate Part 4 HDLs — why an HDL is not a programming language, concurrency, the anatomy of a module, event-driven simulation, Verilog against VHDL, reference cards, and what a testbench is made of PRACTICAL Part 5 Tools and installation Part 6 Fourteen guided tutorials, A to N, at the keyboard Exercises 103 graded exercises with worked solutions Reference The rules, the subset, and the commands, on two pages

How to use this workbook Read a part, then do its tutorial with a terminal open. Exercises marked [H] are hand work — do them on paper first. [C] need the computer. [W] ask you to write something you could defend in a design review. Predict before you measure. Almost every exercise here is worth more if you write down what you expect first. Being wrong and knowing why is the point of a lab; running a command and copying the output is not.

# Part 1 · What RTL Is

## 1.1  The name is the definition

A register transfer level description says two things, and nothing else:

1.  Which registers exist.

2.  What transfers into each one, on each clock edge.

Everything between the registers — the adder, the comparison, the multiplexer — is combinational logic that has one clock period to settle. You never say how long it takes or which gates it uses. You say what value lands in which register on the next edge.

rtl/transfer.v — read this as a table of simultaneous transfers

always @(posedge clk) begin x <= din; // input -> x y <= x + 8'd1; // x, plus logic -> y z <= y << 1; // y, plus logic -> z acc <= acc + z; // acc and z -> acc end

Read that block as a table, not as a sequence. All four assignments read the OLD values of x, y, z and acc, and all four land at the same instant. That is what <= means, and section 1.5 is about why it has to work that way.

### Watch it happen

A single 5 is applied on cycle 0 and never again. It lands in x on the first edge, becomes 6 in y on the second, 12 in z on the third, and reaches acc on the fourth. Nothing moved in between. That is the entire timing model of RTL, and it is why RTL is tractable to reason about where a gate-level netlist is not.

## 1.2  The two kinds of logic, and there is no third

Every digital block ever built is some arrangement of two kinds of logic. Learning to see which one you are looking at is the first skill this topic asks for, because almost every rule that follows applies to one and not the other.

### Combinational

The output is a function of the inputs right now. No clock, no memory, no history. Change an input and the output follows after a propagation delay, and that is the whole story.

assign y = a & b; // continuous assignment always @(*) begin // or a combinational always block y = a & b; // BLOCKING assignment end

The thing that goes wrong here is the inferred latch — a path through the block on which some output is never assigned, so the tool has to build something that remembers. Section 2.3 is about that, and it is the single most common RTL bug there is.

### Sequential

The output depends on the inputs and on the past. A flip-flop samples its input at one instant — the clock edge — and holds that value until the next edge, whatever the input does in between.

always @(posedge clk or negedge rst_n) begin if (!rst_n) q <= 1'b0; // reset appears HERE and nowhere else else q <= d; // NON-BLOCKING assignment end

The things that go wrong here are timing failures: the data arrives too late for the edge (a setup violation) or changes too soon after it (a hold violation). Neither is visible in an RTL simulation, which is why static timing analysis exists — and why it is Topic 6.

The whole of RTL design, in one sentence Deciding what goes in the registers, and what happens between them. That is not a slogan. It is literally what the letters R, T and L stand for, and if you can answer those two questions about a block you can write it.

## 1.3  The synchronous discipline

Almost every rule in this topic — and most of the rules in Topics 5 and 6 — is a consequence of one decision: one clock, one edge, everything. It is worth stating the discipline explicitly, because a design that has drifted out of it fails in ways that are very hard to diagnose.

| The rule | What it means | Why |
| --- | --- | --- |
| one clock edge | every register samples at the same instant | so 'now' means the same thing everywhere |
| one reset policy | sync or async, active high or low — pick one | so nobody has to look it up per module |
| no logic on the clock | no gated clocks, no ripple clocks | so timing analysis has one thing to analyse |
| no latches | assign every output on every path | so timing is edge-to-edge, not level-dependent |
| registered outputs | a block's outputs come out of flip-flops | so a slow path never crosses two blocks |

### What the discipline buys you

▪  Analysable. With one clock and one edge, timing analysis is a finite question: for every path from a flip-flop to a flip-flop, does the data arrive in time? Add a gated clock and the question multiplies.

▪  Composable. Two blocks written to the same discipline can be wired together without a conversation. Two blocks written to different disciplines need one, every time.

▪  Reviewable. A reviewer can read your block for what it computes, because the question of WHEN has already been answered the same way it always is.

▪  Testable. Scan insertion — the technique that makes a chip testable after manufacture — assumes edge-triggered flip-flops on one clock. Latches and gated clocks each need special handling.

You are allowed to break these rules Clock gating and multiple clock domains are real techniques, used in every serious design. What makes them safe is that they are deliberate: chosen, reviewed, constrained and documented. What is not safe is a design that drifted out of the discipline because nobody noticed. You are not allowed to break these rules by accident.

## 1.4  Why anyone designs at this level

|  | Behavioural | RTL | Gate netlist |
| --- | --- | --- | --- |
| you write | the algorithm | registers and transfers | every gate |
| timing | none at all | one clock period per stage | exact, per gate |
| synthesisable | rarely | yes — this is the target | yes, but why would you |
| a 10k-gate design | unbuildable | a few hundred lines | tens of thousands of lines |
| who writes it | architects, in C or SystemC | you | the synthesiser |

RTL is where the trade lands correctly High enough that a human can write and read a real design; low enough that a tool can build it without guessing at your intent. Above RTL you cannot say WHEN things happen. Below it you cannot say anything else. Every industrial digital design of the last thirty years was written at this level, and that is not fashion — it is where the abstraction pays for itself.

## 1.5  Blocking and non-blocking assignment

### The swap, worked through

// NON-BLOCKING - inside always @(posedge clk) // Step 1: read every right-hand side, using the OLD values. // Step 2: update every left-hand side, all at the same instant. a <= b; // reads old b b <= a; // reads old a a and b are SWAPPED. This is what a hardware register does. // BLOCKING - inside always @* // Each statement finishes before the next one starts. a = b; // a is now b b = a; // a is already b, so this does nothing BOTH end up holding b. This is what software does.

Why the rule is absolute rather than stylistic Two clocked blocks using blocking assignments can see each other's half-updated values, and which one wins depends on the order the simulator happens to evaluate them in — an order the language standard deliberately does not fix. Non-blocking assignment exists precisely to make that race impossible. It is not a convention; it is the mechanism.

The mirror-image rule matters too. Non-blocking assignment inside a combinational block is legal, and it makes the block behave like a register in simulation while synthesis builds plain logic — so the simulation and the silicon disagree. Rules L001 and L002 of the linter in this lab catch both directions.

### And what each one actually builds

The argument above is about semantics. Here is the same argument as a measurement. Two files, identical apart from one character per line:

// shift_nb.v - the intended 3-stage shift register always @(posedge clk) begin q[0] <= din; q[1] <= q[0]; q[2] <= q[1]; end // shift_bl.v - the same three lines with = always @(posedge clk) begin q[0] = din; q[1] = q[0]; q[2] = q[1]; end

$ make pitfalls non-blocking version : 0 wrong cycles blocking version : 6 wrong cycles shift_nb (non-blocking) 3 cells 3 flip-flops shift_bl (blocking) 1 cells 1 flip-flops

Three flip-flops against one The blocking version did not build a slower shift register, or a buggy one. It built a different circuit — din races all the way to q[2] in a single clock cycle, because q[0] already holds the new value when line two reads it. It compiled. It simulated. It synthesised. No tool issued a single warning, because nothing illegal was written. This is exactly the class of bug a methodology rule exists to prevent, and rule L001 catches it in about a millisecond.

## 1.6  Four levels of abstraction

Verilog can describe hardware at four levels. The lab writes the same full adder at all four and simulates them together:

// BEHAVIOURAL - you describe the function always @* {cout, sum} = a + b + cin; // DATAFLOW - you describe the Boolean form assign sum = a ^ b ^ cin; assign cout = (a & b) ¦ (b & cin) ¦ (a & cin); // GATE - you name every gate and every wire xor x1 (s1, a, b); xor x2 (sum, s1, cin); and a1 (ab, a, b); and a2 (bc, b, cin); and a3 (ac, a, cin); or o1 (cout, ab, bc, ac); // SWITCH - you place individual transistors pmos p1 (y, vdd, a); nmos n1 (y, gnd, a); // one CMOS inverter

All four were driven with all eight input patterns — exhaustive, since a full adder has only three inputs — and compared against plain arithmetic. Zero mismatches. The level of abstraction changed what was written, not what it does.

### And what a synthesiser makes of each

Two results worth stopping on The behavioural description produced the SMALLEST circuit — five cells against six. It left the tool free to choose the Boolean form, and the tool found a better one than the textbook expression. Dataflow and gate produced the IDENTICAL netlist. Once you have written the Boolean expression you have already committed to the structure; naming the gates adds nothing but typing. And the switch-level description was refused outright. Transistor primitives are for library cells, not for synthesis.

The rule this gives you Write at the highest level that expresses your intent. Every level you descend takes a decision away from the tool and gives it to you — whether or not you wanted it. The tool is better than you are at choosing a gate mix, balancing a logic tree, and doing it again consistently at 3 a.m. It is not better than you are at deciding how many cycles the job takes, where the registers go, what is shared, or what the interface looks like. Give it the first list; keep the second.

## 1.7  Simulation shows; proof settles

Eight patterns was exhaustive for a full adder. It stops being possible at about thirty inputs, and real designs have thousands — so the lab makes the same claim a different way. An equivalence check builds a miter: both designs fed the same inputs, their outputs compared, and an assertion that the comparison always holds. A SAT solver then tries to find any input pattern that breaks it.

$ make prove fa_behav vs fa_dataflow EQUIVALENT (proved, 94 SAT variables) fa_behav vs fa_gate EQUIVALENT (proved, 94 SAT variables) fa_dataflow vs fa_gate EQUIVALENT (proved, 100 SAT variables) and the same checker, on a full adder with one term missing: fa_behav vs fa_broken NOT EQUIVALENT

That last line is the important one fa_broken.v has one term missing from the carry, so it is wrong for exactly one input pattern in eight. A random test could easily miss it. The solver cannot. A checker that cannot fail is not evidence of anything. You have to watch it catch a real bug before you can trust it on a design you cannot check by hand.

## 1.8  The running example

One design is carried through the rest of this workbook: a four-bit counter with an asynchronous reset, an enable and a terminal-count output. It is small enough to hold in your head and real enough to have every decision a real design has.

rtl/counter4.v

module counter4 ( input clk, input rst_n, // asynchronous, active LOW input en, output reg [3:0] count, output tc // one cycle high at 15 ); always @(posedge clk or negedge rst_n) begin if (!rst_n) count <= 4'd0; else if (en) count <= count + 4'd1; end assign tc = en & (count == 4'd15); endmodule

### Four decisions are visible in fourteen lines

▪  The reset is asynchronous. That is why negedge rst_n is in the sensitivity list. A synchronous reset would not be — it would just be the first test inside the block.

▪  The reset is active low. That is why the port is named rst_n and the test is !rst_n. Naming the polarity in the signal name is a convention worth keeping: it makes a wrong connection visible in the instantiation.

▪  Reset is tested first. It therefore wins over the enable. Written the other way round, a reset arriving while en was low would be ignored.

▪  There is no else on the enable. This is deliberate, and it is the one case where a missing else is correct: in a CLOCKED block, no assignment means hold, which is exactly what a flip-flop does. The same omission in a combinational block would build a latch.

### The numbers, worked out

Run the arithmetic before you run the simulator. At 100 MHz the clock period is 10 ns, the counter has 2⁴ = 16 states, so one full cycle takes 160 ns and tc pulses at 6.25 MHz for exactly one clock period. Every one of those numbers is predictable from the source without running anything, and being able to do that is most of what design review is.

Where the running example goes next Part 3 turns its controller into a state machine, and reuses the counter as the timer inside the traffic-light controller. Part 3 again parameterises it as counter_n and instantiates it twice to build a prescaler. Tutorial N runs it through the whole front end: spec, lint, simulation, synthesis, gate-level simulation, comparison and formal proof.

Part 1 self-check 1. What two things does an RTL description state? 2. What are the only two kinds of digital logic? 3. Name three things the synchronous discipline buys you. 4. Explain what happens to a and b in a<=b; b<=a; and in a=b; b=a; 5. How many flip-flops did the blocking shift register build, and why? 6. Name the four levels of abstraction, highest first. 7. Which level produced the smallest netlist, and why? 8. Why did dataflow and gate produce identical netlists? 9. Why does the lab include a deliberately broken full adder? 10. In counter4, why is there no else on the enable?

# Part 2 · The RTL Design Process and Methodology

## 2.1  The flow

Every arrow points both ways in practice. A timing failure sends you back to the RTL; a synthesis surprise sends you back to the micro-architecture; a specification that turns out to be ambiguous sends you back further than anyone would like.

### Executed, not described

A flow drawn on a slide is a picture of arrows. make flow runs the same arrows on a real design and shows what each stage produces — because a stage that produces no evidence is a stage nobody can tell you skipped.

$ make flow STAGE 1 SPECIFICATION 4 sentences, written before the RTL STAGE 2 LINT 0 issues STAGE 3 RTL SIMULATION 18 cycles; wraps at 15, tc correct STAGE 4 SYNTHESIS 12 cells STAGE 5 GATE SIMULATION the netlist, same stimulus STAGE 6 COMPARE IDENTICAL on all 18 cycles STAGE 7 PROVE Equivalence PROVEN by induction All seven stages passed.

Stage 7 is the one worth understanding Stage 6 says the RTL and the netlist agree on the 18 cycles that were tested. Stage 7 proves they agree on EVERY input sequence, by induction, without enumerating any of them. The script stops at the first stage that fails. That is what makes it a methodology rather than a diagram.

## 2.2  The synthesisable subset

"Verilog is not a programming language" stays a slogan until you watch a synthesiser refuse to build something. The lab runs eleven constructs through Yosys and reports what came out.

### Three rows worth discussing

| Row | What happened | Why it matters |
| --- | --- | --- |
| a / b against a / 4 | 371 cells against 0 | same operator; the difference is entirely what you divided BY. A constant power of two is a rename of wires. |
| #5 delay in RTL | synthesised to 1 NOT gate | the delay silently vanished. Silicon cannot wait five time units; the delay a gate has is set by the library and the layout. |
| initial block | accepted, 10 cells | Yosys targets FPGAs, where the bitstream really does initialise the flops. An ASIC flow would not — this is how code that works on an FPGA fails on an ASIC. |

The habit this table is for Before you use a construct you are unsure about, synthesise a ten-line module containing only that construct and read what came out. It takes two minutes and it is the only way to actually know, as opposed to remembering what someone told you about a different tool version.

## 2.3  The inferred latch

When en is 0 the code never says what y should be — so the tool must build something that remembers the old value. That something is a level-sensitive latch, which you did not ask for and almost certainly do not want.

|  | A flip-flop | A latch |
| --- | --- | --- |
| captures | on the clock EDGE | while the enable is HIGH |
| is transparent | never | for the whole enable window |
| timing analysis | one clean check per edge | time-borrowing; much harder to analyse |
| in a scan chain | standard | usually needs special handling |
| you asked for it | yes, by writing posedge | no — you forgot an else |

Why it is worse than an error It is not an error at all. The tool builds it, mentions it in a log nobody reads, and hands you a design with a memory element in the middle of what you thought was combinational logic — which then has its own timing requirements and quietly complicates your static timing analysis. The rule is mechanical: in a combinational block, assign every output on every path. An if needs an else; a case needs a default. If you would rather not write them all out, assign a default at the top of the block and let later assignments override it.

## 2.4  When simulation and silicon disagree

This is the worst bug in the topic, because it is the only one that cannot be found by testing — the thing you are testing is not the thing that will be built. The lab drives the RTL and the netlist Yosys produced from it with identical stimulus:

$ make mismatch change a (list wakes) a=1 b=0 RTL y=0 NETLIST y=0 change b (list ASLEEP) a=1 b=1 RTL y=0 NETLIST y=1 <-- DISAGREE disagreements: 1 of 6 Your testbench was verifying a circuit that does not exist.

| Cause | What simulation does | What synthesis does |
| --- | --- | --- |
| incomplete sensitivity list | the block sleeps; the output goes stale | ignores the list; builds the logic |
| # delays in RTL | waits the specified time | ignores them entirely |
| initial block (ASIC) | sets the value at time zero | ignores it; the flop powers up unknown |
| blocking in a clocked block | result depends on evaluation order | builds one specific circuit |

All four have the same cure Use always @* and never a hand-written sensitivity list. No delays in RTL. A real reset instead of an initial block. Non-blocking in clocked blocks. Then lint for all four, so that nobody has to remember. SystemVerilog's always_comb goes further and makes the tool check it for you.

## 2.5  Seven coding rules, and a tool that checks them

Methodology degenerates into a list of good intentions unless something checks it. Every rule above is one a reviewer would otherwise have to remember, on every file, for ever — exactly the kind of job people are bad at and programs are good at. Every real RTL team lints before simulation, because a lint error costs seconds and the bug it prevents can cost a silicon revision.

### Is the linter telling the truth?

Rules L005 and L006 claim that a missing else or a missing default infers a latch. That is a claim about what a synthesiser will do, so the synthesiser settles it:

$ make lintcheck file linter says yosys says verdict s03_latch latch latch agree s04_incomplete_sens no latch no latch agree s14_latch_case latch latch agree ... 10 files, 0 disagreements L005 and L006 are not opinions - they predict what gets built.

None of these are style preferences. Every one exists because breaking it produces a design that simulates differently from the way it is built — and that class of bug survives every test you write.

## 2.6  Micro-architecture: the decisions RTL cannot make for you

This is where the engineering is A synthesiser is better than you are at choosing gates. It is not better than you are at any of the rows in that table, and it will not warn you that you chose badly — it will faithfully build what you asked for. Make these decisions on paper, before the first line of RTL.

## 2.7  Writing RTL someone else can use

Module 2's fourth terminal outcome asks for reusable IP by name. Reuse is not something you add to a design afterwards; it is a set of decisions taken while writing it. The test is simple: could a colleague drop this into a different design, next year, without asking you anything? If the answer needs a conversation, it is not reusable yet.

## 2.8  Coding style: one function, three ways

A four-to-one multiplexer can be written as a conditional expression, as a case statement, or as an if/else chain. The usual classroom claim is that the optimiser flattens the difference, so you should write whichever reads best. Half of that is right.

$ make mux patterns checked : 64 mismatches : 0 mux4_assign vs mux4_case EQUIVALENT (proved, 92 SAT variables) mux4_assign vs mux4_if EQUIVALENT (proved, 76 SAT variables) mux4_assign 3 cells 0 flip-flops mux4_if 6 cells 0 flip-flops mux4_case 10 cells 0 flip-flops

Equivalent is not the same as identical The SAT proof answers exactly one question: do these three descriptions compute the same Boolean function? They do. It says nothing whatever about area, timing or power — those are separate questions with separate tools. The conditional expression hands sel[1] and sel[0] straight to the multiplexer selects. The case and if/else versions ask the tool to build equality comparators against 2'b00, 2'b01 and so on, and then to work out for itself that those comparisons ARE the select bits. On Yosys 0.33 it gets part of the way.

So which do you write? Whichever reads best to the next person — usually the case statement once there are more than three branches — and then measure, on your own tool, if the block turns out to be on a critical path. A different synthesiser may well close the gap. Folklore about what optimisers do is the least reliable kind of knowledge in this field, and it is cheap to replace with a measurement.

Part 2 self-check 1. Name the first four stages of the flow, and say what evidence each produces. 2. Why lint before simulating rather than after? 3. What exactly builds a latch? 4. a / b cost 371 cells and a / 4 cost 0. Explain. 5. Why is an incomplete sensitivity list worse than a syntax error? 6. Name three decisions synthesis will not make for you. 7. Does a formal equivalence proof say anything about area? 8. Three mux styles gave 3, 6 and 10 cells. Explain why.

# Part 3 · The Patterns Every Block Is Built From

Parts 1 and 2 were about what RTL is and what you are allowed to write. This part is about what real designs are actually made of. Three structures account for almost all synthesisable RTL, and once you can recognise them most designs stop looking unfamiliar.

## 3.1  Datapath and controller

A UART, a cache controller, a DMA engine, a GPU shader core — different sizes, the same two halves.

▪  The datapath holds and transforms data: registers, adders, multiplexers, counters. It is wide, it is most of the area, and it makes no decisions at all.

▪  The controller is a finite state machine that decides WHEN each datapath element loads, clears or holds. It is narrow, it is a handful of gates, and it contains all of the behaviour.

They talk over two thin bundles of wires: control signals downward, status signals upward. That is the entire interface.

### The worked example: accumulate N samples

Assert start with the sample count on n; feed one sample per clock on data; done pulses when the sum is valid. The controller is three states.

rtl/datapath_ctrl.v, abridged

// ---- the CONTROLLER decides. Narrow. No data passes through it. always @(*) begin acc_clr = 0; acc_en = 0; cnt_ld = 0; cnt_dec = 0; done = 0; case (state) S_IDLE : if (start) begin acc_clr = 1; cnt_ld = 1; end S_RUN : if (!cnt_done) begin acc_en = 1; cnt_dec = 1; end S_DONE : done = 1; default: ; endcase end // ---- the DATAPATH holds and transforms. Wide. It decides nothing. always @(posedge clk or negedge rst_n) begin if (!rst_n) sum <= 0; else if (acc_clr) sum <= 0; else if (acc_en) sum <= sum + data; end

$ make dpctrl cycle data ¦ clr en ld dec done ¦ sum 0 0 ¦ 1 0 1 0 0 ¦ 0 1 10 ¦ 0 1 0 1 0 ¦ 0 2 20 ¦ 0 1 0 1 0 ¦ 10 ... 7 0 ¦ 0 0 0 0 0 ¦ 157 8 0 ¦ 0 0 0 0 1 ¦ 157 golden total : 157 hardware sum : 157 PASS - controller sequenced the datapath correctly

### What the split costs, measured

| module | cells | flip-flops | what it contains |
| --- | --- | --- | --- |
| accum_ctrl | 10 | 2 | three states, five control outputs |
| accum_datapath | 145 | 24 | a 16-bit accumulator and an 8-bit down-counter |
| accum_top | 156 | 26 | both, plus the wires between them |

Six per cent of the cells, one hundred per cent of the behaviour That ratio is the whole argument for the split. Over the life of a design you will re-time, widen and pipeline the expensive half many times — and each time, the half that decides what happens stays exactly as it was. It also means the part that is hardest to get right is the part small enough to read in one sitting. A ten-cell controller can be reviewed line by line; a 145-cell datapath cannot, and does not need to be.

### Which signals go where

| Signal | Direction | Why it belongs there |
| --- | --- | --- |
| acc_clr, acc_en | control, down | the controller decides when to clear and accumulate |
| cnt_ld, cnt_dec | control, down | the controller decides when to load and decrement |
| cnt_done | status, up | the datapath knows the count; the controller only asks |
| sum | data, out | never crosses into the controller — it does not need it |
| start, done | handshake | the block's interface, not an internal signal |

The row worth arguing about is cnt_done. It would be possible for the controller to hold the counter itself and test it directly. Do that and the controller becomes width-dependent: change the maximum sample count and you edit the state machine. Keeping the counter in the datapath and passing up a single status bit means the controller never changes at all.

## 3.2  The finite state machine

A state machine is the standard way to write anything that has to happen in a sequence. Almost every RTL group writes them in the same three blocks, and the reason is not tradition.

### Block 1 — the state register

always @(posedge clk or negedge rst_n) begin if (!rst_n) state <= S_IDLE; else state <= next_state; end

Sequential. Non-blocking assignment. This is the only sequential logic in the machine, so it is the only place a clock or a reset appears — which makes it the only block whose timing you have to think about.

### Block 2 — the next-state logic

always @(*) begin next_state = state; // <-- THE DEFAULT ASSIGNMENT case (state) S_IDLE : if (din) next_state = S_1; S_1 : if (din) next_state = S_1; else next_state = S_10; ... default: next_state = S_IDLE; endcase end

Combinational. Blocking assignment. A pure function of the current state and the inputs, with no notion of time in it at all.

### Block 3 — the output logic

assign det = (state == S_101); // Moore: state only // or, when several outputs are decoded: always @(*) begin main_light = RED; side_light = RED; // defaults again case (state) S_MAIN_GREEN : begin main_light = GREEN; side_light = RED; end ... endcase end

The default assignment is the whole trick Look again at blocks 2 and 3. Not one of those if statements has an else, and not one of them needs one — the output was already written, unconditionally, on the first line of the block. No path through either block leaves anything unassigned, so no latch can be inferred, whatever the case statement leaves out. This is the recommended way to write a next-state or output block, and it is why the lab linter had to learn to recognise it. A naive latch rule flags every one of those if statements; Yosys builds a latch in none of them. make lintcheck puts the two opinions side by side on sixteen designs, and a linter that cries wolf gets switched off — which makes it worse than no linter at all.

### Why not one block?

You can write a working state machine in a single clocked block, with the case statement inside it. Nothing about it is illegal, and for a very small machine it is shorter. Here is what you give up:

|  | one block | three blocks |
| --- | --- | --- |
| works? | yes | yes |
| where is the reset? | mixed in with the logic | block 1, and nowhere else |
| where is the clock? | wrapped around everything | block 1, and nowhere else |
| outputs are | registered, always | your choice — Moore or Mealy |
| reviewing a change | you re-read the timing every time | blocks 2 and 3 have no timing to re-read |
| adding a state | one place, easy to miss a branch | two places, both of them a case statement |

One block is not wrong. Three blocks are easier to be right in, and over a project that is the same thing.

## 3.3  Moore and Mealy

There is exactly one structural difference between the two: whether the output logic is allowed to see the input. Everything else follows from it.

### The same detector, twice

Both machines below detect the pattern 1-0-1 arriving one bit per clock, counting overlaps — so the stream 1 0 1 0 1 contains two matches, not one.

// MOORE - the output is decoded from the state alone assign det = (state == S_101); // MEALY - the output depends on the state AND the current input assign det = (state == S_10) && din;

### The difference, measured

$ make fsm cycle din mealy exp moore exp 3 1 1 1 0 0 <- match 4 1 0 0 1 1 6 1 1 1 0 0 <- match 7 0 0 0 1 1 matches in the stream : 5 mismatches vs golden : 0 PASS - same language, Moore trails Mealy by one cycle

The golden model is computed from the stimulus stream itself, not from either machine — so a bug in one of them appears as a mismatch rather than as two machines confidently agreeing on the wrong answer.

|  | Moore | Mealy |
| --- | --- | --- |
| output depends on | state only | state and input |
| output appears | one cycle after the input | the same cycle |
| output glitches | no — decoded from registers | yes — inherits the input's glitches |
| states needed | usually one more | usually one fewer |
| measured here | 13 cells, 2 flip-flops | 14 cells, 2 flip-flops |

Choosing between them Moore when the output leaves the block, drives other logic, or is timing critical. It comes out of a decode of registered bits, so it is clean, predictable and easy to constrain. Mealy when the cycle genuinely matters — a handshake that must be answered immediately, or a pipeline where one more cycle of latency costs throughput you cannot spare. Either way, register it at the boundary. A Mealy output leaving your block hands the next designer a combinational path they did not ask for. Put a flip-flop on it — which turns it back into Moore.

One piece of folklore is worth discarding here. “Mealy needs fewer states” is true and almost never the reason to pick it. In this example it saved one flip-flop's worth of nothing — both machines needed two — and cost one cell more. The real trade is the cycle against the glitch-free output, and that is a system question, not a coding one.

## 3.4  State encoding

The state names are just numbers you chose. Choosing them differently gives the same behaviour and different hardware.

// binary // one-hot localparam [1:0] S_IDLE = 2'b00, localparam [3:0] S_IDLE = 4'b0001, S_1 = 2'b01, S_1 = 4'b0010, S_10 = 2'b10, S_10 = 4'b0100, S_101 = 2'b11; S_101 = 4'b1000;

| encoding | flops | cells | the property | where it wins |
| --- | --- | --- | --- | --- |
| binary | 2 | 13 | fewest flip-flops | ASIC, many states, area-critical |
| one-hot | 4 | 30 | decode is one wire | FPGA, speed-critical, few states |
| gray | 2 | — | one bit changes per transition | the state crosses a clock domain |

Read the measurement before repeating the folklore On this generic gate library one-hot came out BIGGER, not smaller: four states need four flip-flops instead of two, and the next-state logic has to drive four bits instead of two. Thirty cells against thirteen. One-hot is still usually right on an FPGA, where a flip-flop beside each lookup table is effectively free and the win is the short decode path — and it grows more attractive as the number of states rises, because binary decode logic grows faster than the extra registers do. Both statements are true. Neither is a rule. Measure your own target.

## 3.5  A controller with a timer, and how to check it

A traffic-light controller for a two-road junction, with a car sensor on the minor road. The main road stays green until a car appears, then the sequence runs main-yellow, side-green, side-yellow and back. Green lasts six cycles and yellow two.

Where the timer lives, and why it is not a state A six-cycle green phase does not mean six states, and a six-hundred-cycle one certainly does not. The count lives in a down-counter — a datapath element — and the state machine only ever asks it one question: are you at zero yet? That is section 3.1 again, in the smallest design in this topic. Measured: 75 cells and 10 flip-flops, of which 2 are the state and 8 are the timer.

### Checking a property instead of reading a waveform

A traffic-light controller has safety requirements that are easy to state and tedious to check by eye. Write them once as code and the simulator checks them on every cycle of every run:

// P1 : the two roads are never green at the same time if (main_light == GREEN && side_light == GREEN) begin $display("*** P1 VIOLATED at cycle %0d", cycles); errors = errors + 1; end // P2 : a green never goes straight to red - yellow comes first if (prev_main == GREEN && main_light == RED) begin $display("*** P2 VIOLATED at cycle %0d", cycles); errors = errors + 1; end

$ make fsm cycles checked : 40 property violations : 0 PASS - mutual exclusion and yellow-before-red both hold

This is the cheap half of formal verification, available in any simulator, in about ten lines. It scales to runs far longer than anyone would page through by hand, and — unlike a waveform you inspected once — it keeps working after you change the design. The expensive half, proving the property holds for every possible input rather than the ones you tried, is what make prove does with a SAT solver.

## 3.6  From a module to an IP

Module 2's terminal outcomes ask you to design and develop IPs, and to characterise reusable ones. Reusable means one source file that covers a family of widths and depths — not fourteen copies with the numbers edited. Three language features do all the work, and none of them exists in the hardware.

| Feature | What it is | When it disappears |
| --- | --- | --- |
| parameter | a compile-time constant the instantiator can override | elaboration |
| hierarchy | a module instantiated inside another module | flattening, if the tool flattens at all |
| generate | a compile-time loop that builds N copies of a structure | elaboration |

rtl/reuse.v

module delayline #(parameter W = 8, parameter N = 4) (input clk, rst, en, input [W-1:0] din, output [W-1:0] dout); wire [W-1:0] tap [0:N]; // one more element than stages assign tap[0] = din; genvar k; generate for (k = 0; k < N; k = k + 1) begin : stage preg #(.W(W)) u_reg (.clk(clk), .rst(rst), .en(en), .d(tap[k]), .q(tap[k+1])); end endgenerate assign dout = tap[N]; endmodule

$ make reuse delayline #(W=8, N) synthesised at four different depths: N = 1 8 cells 8 flip-flops N = 2 16 cells 16 flip-flops N = 4 32 cells 32 flip-flops N = 8 64 cells 64 flip-flops

The for loop is not a loop It is an instruction to the elaborator: make N instances of preg and wire stage k's output to stage k+1's input. After elaboration there are N registers and no loop anywhere in the design. The measured table is the proof: eight flip-flops per stage, exactly N stages, linear. The same run instantiates one parameterised counter twice, at different widths, to build a prescaler — and checks that it divides by exactly 16. That is all “hierarchy” means: a wire between two instances.

Part 3 self-check 1. What are the two halves of almost every block, and what crosses between them? 2. Why keep the sample counter in the datapath rather than the controller? 3. Name the three blocks of a state machine and say what each one is for. 4. Why does a default assignment at the top of a block prevent a latch? 5. What does the output logic read in a Moore machine? In a Mealy machine? 6. How much later does a Moore output appear, and why is that ever acceptable? 7. Did one-hot encoding cost more or less than binary here, and why? 8. Why is the traffic light's phase timer not implemented as states? 9. Write the property that says two roads are never both green. 10. Does a generate loop appear anywhere in the netlist?

# Part 4 · Hardware Description Languages

## 4.1  An HDL is not a programming language

It looks like one, and that resemblance is the single biggest source of beginner bugs. You are not writing instructions for a machine to follow; you are writing a description of a machine, which a tool will then build. The text is a blueprint, not a recipe.

Which is why "it compiles" means almost nothing here, and why a construct can be perfectly legal Verilog and still have no hardware meaning at all — as the subset table in Part 2 measured.

## 4.2  Everything happens at once

Write those three assign lines in any order you like and the circuit is identical, because you did not write a sequence — you wrote three facts about three pieces of hardware that all exist together, permanently.

The one place order does matter is inside a single always block using blocking assignments, where statements do run in order, like software. That is exactly why mixing = and <= in one block is confusing enough to be a lint rule.

## 4.3  The anatomy of a module

The word that confuses everyone: reg reg does NOT mean register. It means "this signal is assigned inside a procedural block". A reg assigned in an always @* block becomes pure combinational logic with no storage whatsoever. What actually creates a flip-flop is assigning inside always @(posedge clk) — nothing else. SystemVerilog renamed reg to logic precisely to end this confusion.

## 4.4  How a simulator runs an HDL

Nothing runs continuously. The simulator jumps from event to event: a signal changes, everything sensitive to it wakes, they are all evaluated, non-blocking results are queued, the queue is applied at once, and the process repeats until nothing more changes — only then does time advance.

Step 3 is why RTL has coding rules at all The order in which woken blocks are evaluated is genuinely unspecified by the standard. Two clocked blocks using blocking assignments can see each other's half-finished work, and which one wins may differ between simulators, or between runs of the same simulator. Non-blocking assignment exists to make that impossible: every right-hand side is read before any left-hand side is updated, so evaluation order cannot affect the result.

## 4.5  Verilog and VHDL

None of those differences are about hardware. Both describe registers, combinational logic and hierarchy; both synthesise to the same gates. An engineer who understands RTL can read the other one after an afternoon; an engineer who has only memorised syntax can read neither.

### The same designs, in both languages, actually run

$ make langs === Verilog: iverilog === cycle 14 count=1111 tc=1 cycle 15 count=0000 tc=0 === VHDL: ghdl === cycle 14 count=1111 tc=1 cycle 15 count=0000 tc=0 === diff of the two transcripts === IDENTICAL over all 18 cycles - including the wrap and the terminal count. === the same '101' detector, both languages === IDENTICAL over all 17 cycles, detections included.

Not "they look similar". The logs were compared line by line by diff, through two different simulators, and there was nothing to report.

The state machine is the more interesting of the two, because it is where the languages genuinely differ. In Verilog the states are numbers you chose; in VHDL they are a type:

// Verilog localparam [1:0] S_IDLE = 2'd0, S_1 = 2'd1, S_10 = 2'd2, S_101 = 2'd3; -- VHDL type state_t is (S_IDLE, S_1, S_10, S_101); signal state, next_state : state_t;

The VHDL version cannot be assigned a value that is not one of those four — the analyser rejects it — and it also rejects a case statement that is not exhaustive. The Verilog version will accept state <= 2'd7 and nobody will complain until silicon. That is the trade in one sentence: VHDL catches more mistakes at compile time and costs more keystrokes; Verilog is faster to write and trusts you more than it should.

## 4.6  Reference cards, and what a testbench is made of

The two pages that follow are the whole of the language you need for this topic — not the whole of Verilog or VHDL, which are both far larger, but the part that synthesises. Keep them beside you until you no longer need them.

### Verilog to VHDL, line for line

### The anatomy of a testbench

Topic 5 covers verification properly. Six parts are enough to finish Topic 2, and the fourth is the one people leave out:

1.  Clock. always #5 clk = ~clk; — nothing else generates it.

2.  Reset. Asserted before anything else, and released away from the sampling edge on purpose.

3.  Stimulus. Driven on the inactive edge, sampled before the active one, so the design never sees a value changing at the instant it samples.

4.  Golden model. The expected answer, computed from the STIMULUS and never from the design under test.

5.  Checker. The line that compares them and counts. This is what makes a testbench self-checking.

6.  Verdict. One line a human can read without opening a waveform.

A testbench without a golden model is not a test If the expected answer comes from the design, the test passes whatever the design does. Every checker in this lab computes its expectation from the stimulus — which is the only reason it was able to print blocking version : 6 wrong cycles at all. Eyeballing a waveform is not a checker either. It does not scale past about ten cycles, and it never runs twice.

## 4.7  Which one should you learn?

Part 4 self-check 1. Give the biggest single difference between an HDL and a programming language. 2. Does the order of assign statements matter? Why not? 3. What does reg actually mean, and what actually creates a flip-flop? 4. Why does the standard leave evaluation order unspecified, and what saves you from it? 5. Your colleague only knows VHDL. What can they read of your Verilog, and what will they need explained?

# Part 5 · Tools and Installation

## 5.1  Which tool answers which question

Yosys is the interesting one It is the only free tool in that list that will both synthesise your RTL and PROVE that the netlist it produced is equivalent to what you wrote. That combination is what makes the seven-stage flow in Part 2 runnable on a laptop.

## 5.2  Installing

Debian, Ubuntu or WSL2

sudo apt update sudo apt install yosys iverilog gtkwave python3 sudo apt install ghdl # only for the VHDL comparison # verify: yosys -V && iverilog -V && ghdl --version

On Windows, install WSL2 first (wsl --install from an elevated PowerShell) and run the same lines inside it. Without ghdl, every target except make langs still works.

### The vendor tools the syllabus names

An honest note about the vendor tools Vivado and ModelSim are not installed in the environment these materials were built in. Every number quoted in this workbook came from iverilog, ghdl and yosys, and every one is reproducible with make. The commands shown for Vivado and ModelSim come from the vendor documentation and are labelled as not executed here. Tutorial N asks you to run them and record what you get — which is itself a useful exercise in not trusting a number you did not produce.

## 5.3  What the lab does with your RTL

Four different questions about one piece of code: does it follow the rules, does it do what the spec says, what will actually be built from it, and is that thing still the design you wrote? A methodology is exactly the discipline of asking all four, every time.

# Part 6 · Fourteen Guided Tutorials

Before you start Install the toolchain and open a terminal in Topic2_Lab/. Type the commands; do not paste them. The point of a tutorial is the twenty seconds between typing something and understanding why it did what it did. Each tutorial ends with a Checkpoint — a specific thing you should be able to show or say. If you cannot, stop and find out why before moving on.

## Tutorial A · What RTL means  (1 hour)

Open rtl/transfer.v and read the always block as a table of simultaneous transfers. Then, before running anything, fill in this table on paper for the first three cycles:

cycle din ¦ x y z ¦ acc ------------+--------------------+------ 0 5 ¦ ? ? ? ¦ ? 1 0 ¦ ? ? ? ¦ ? 2 0 ¦ ? ? ? ¦ ?

Now run make transfer and compare.

The row people get wrong On cycle 0, y is 1 — not 6. The assignment y <= x + 1 used the OLD value of x, which was still 0 when the edge arrived. The 5 only reaches x on that same edge, so it cannot reach y until the next one. If you predicted 6, you were reading the block as a sequence. Read it as a table.

Checkpoint A You can state the two things an RTL description says, and explain why y is 1 rather than 6 on cycle 0 without looking it up.

## Tutorial B · The abstraction ladder  (2 hours)

### Step 1 — read all four

Open the four files in ladder/ side by side. They are the same full adder. Note how much longer each one gets as you descend, and note that only the top one says what the circuit is FOR.

### Step 2 — run them together

$ make ladder a b cin ¦ golden ¦ behav dataflow gate switch ---------+--------+-------------------------------- 0 0 0 ¦ 0_0 ¦ 0_0 0_0 0_0 0_0 ... 1 1 1 ¦ 1_1 ¦ 1_1 1_1 1_1 1_1 patterns applied : 8 of 8 (exhaustive - every possible input) mismatches : 0 PASS - all four descriptions are the same circuit.

### Step 3 — synthesise each one and predict first

Before running it: which level do you expect to produce the fewest cells? Most people say gate, on the grounds that it is the most specific. Write your prediction down, then run the synthesis step and look at section 1.4 of this workbook.

### Step 4 — the transistor level

Open ladder/fa_switch.v. Count the transistors in nand2_sw: two pmos in parallel pulling up, two nmos in series pulling down. Work out why an AND needs six transistors and a NAND needs four — and therefore why CMOS libraries are full of NANDs and NORs rather than ANDs and ORs.

Checkpoint B All four simulate identically; you predicted which level would synthesise smallest and can explain the answer; and you can say why dataflow and gate produced the same netlist.

## Tutorial C · Proof, not just testing  (2 hours)

$ make prove fa_behav vs fa_dataflow EQUIVALENT (proved, 94 SAT variables) fa_behav vs fa_gate EQUIVALENT (proved, 94 SAT variables) fa_dataflow vs fa_gate EQUIVALENT (proved, 100 SAT variables) fa_behav vs fa_broken NOT EQUIVALENT

### Step 1 — understand what was proved

Read scripts/equiv.sh. It builds a miter, asserts the outputs always match, and asks a SAT solver to find a counter-example. "No model found" is the solver saying: I could not construct one, and I looked everywhere.

### Step 2 — break it yourself

Open ladder/fa_broken.v. It is missing (a & cin) from the carry. Work out on paper which single input pattern that makes wrong, then remove a different term and predict the new failing pattern before you run the check again.

### Step 3 — the limit

A full adder has 8 input patterns, so exhaustive simulation was complete. Work out how many patterns a 32-bit adder has, and how long it would take to simulate them all at a million per second. That number is why equivalence checking exists.

Checkpoint C You have watched the checker prove three equivalences and catch one real bug, and you can explain why exhaustive simulation stops being an option well before a design gets interesting.

## Tutorial D · The synthesisable subset  (2 hours)

Open each file in subset/ and write down, before running anything, whether you expect it to synthesise and roughly how many cells you expect. Then:

$ make subset construct synth cells latch what the tool said s03_latch OK 1 YES inferred a LATCH: $_DLATCH_P_ s08_whileloop REFUSED - - "only allowed in constant functions" s10_divide OK 371 no a full combinational divider s11_shift OK 0 no no logic at all - just wires

### Try these

▪  Change s10_divide to divide by 8 instead of by b. Predict the cell count first.

▪  Change it to divide by 3 — a constant, but not a power of two. Predict again, and explain the result.

▪  Write a twelfth construct of your own that you believe will be refused, and check.

▪  Add (* keep *) to a signal in s01 and see what changes.

Checkpoint D You predicted at least eight of the eleven rows correctly, and for the ones you got wrong you can say what you had assumed and why it was false.

## Tutorial E · Simulation against silicon  (1 hour)

$ make mismatch change a (list wakes) a=1 b=0 RTL y=0 NETLIST y=0 change b (list ASLEEP) a=1 b=1 RTL y=0 NETLIST y=1 <-- DISAGREE disagreements: 1 of 6

Only one of six stimulus steps disagreed. Work out why the other five did not — this matters, because it is exactly why the bug survives casual testing.

### Try these

▪  Change the sensitivity list to always @(b) instead. Which steps disagree now?

▪  Change it to always @*. Confirm the disagreement count goes to zero.

▪  Write the same bug into a clocked block. Does it still mismatch? Why not?

Checkpoint E You can explain to someone who has not seen it, in three sentences, why an incomplete sensitivity list is worse than a syntax error.

## Tutorial F · The coding rules  (2 hours)

$ make lint --- files written the way the methodology asks: --- clean - no rule violations (x4) --- and files that break the rules on purpose: --- line 7 L001 blocking (=) in a clocked block - use <= line 7 L003 = and <= mixed in one always block line 13 L004 explicit sensitivity list - use always @* line 14 L006 case with no default - infers a latch $ make lintcheck 10 files · linter and Yosys agree on every one · 0 disagreements

### Step 1 — read the tool

Open tools/rtl_lint.py. It is a few hundred lines of regular expressions and you can read all of it. Find the function that counts blocking and non-blocking assignments and satisfy yourself that the two counts are disjoint.

### Step 2 — add a rule

Add rule L008 of your own choosing. Candidates worth considering: a clocked block with more than one clock; a module with no ports; a case statement over a signal wider than four bits with fewer than sixteen branches and no default. Write the rule, write a file that breaks it, and write a file that does not.

### Step 3 — break the linter

Find a file that the linter gets wrong — either a false positive or a missed violation. This is not hard, because it is regular expressions and not a parser. Then write down why a regular expression cannot fix the case you found.

Checkpoint F Your new rule fires on the file that breaks it and stays silent on the file that does not; and you have found at least one case the linter gets wrong and can explain why.

## Tutorial G · Coding style  (1 hour)

Open rtl/mux4_styles.v. Three modules, one function. Before running anything, write down which you think will produce the fewest cells, and why.

$ make mux patterns checked : 64 mismatches : 0 mux4_assign vs mux4_case EQUIVALENT (proved, 92 SAT variables) mux4_assign 3 cells mux4_if 6 cells mux4_case 10 cells

### Try these

▪  Write a fourth style — a for loop over the data bits, say — and predict where it lands before you measure.

▪  Rewrite mux4_case using sel as a bit index (y = d[sel];) and see whether the tool now matches mux4_assign.

▪  Explain in two sentences why an equivalence proof cannot tell you which of the three to ship.

Checkpoint G You can state precisely what a formal equivalence proof does and does not tell you, and you have measured one case where coding style reached the netlist.

## Tutorial H · The two pitfalls  (2 hours)

These two account for most of the bugs a new RTL engineer files. Neither is illegal; neither produces a warning.

### Step 1 — blocking assignment in a clocked block

Open pitfalls/shift_nb.v and pitfalls/shift_bl.v. They differ by one character per line. Fill in the table below by hand for the first six cycles of the stimulus, THEN run the lab.

$ make pitfalls non-blocking version : 0 wrong cycles blocking version : 6 wrong cycles shift_nb (non-blocking) 3 cells 3 flip-flops shift_bl (blocking) 1 cells 1 flip-flops

### Step 2 — the inferred latch

Synthesise subset/s03_latch.v and subset/s14_latch_case.v, and confirm the $_DLATCH_ cell in each. Then fix each one — an else in the first, a default in the second — and confirm it disappears.

### Try these

▪  Explain in two sentences why no tool warned about shift_bl.

▪  Write a combinational block with three outputs where only one is latched, and predict the cell count before synthesising.

▪  Take one of your own files from Tutorial F and check it for both pitfalls.

Checkpoint H You can look at a clocked block and say what it will BUILD rather than what it appears to say, and you can spot an inferred latch in code before a tool tells you about it.

## Tutorial I · State machines  (3 hours)

The longest tutorial in the topic, and the most useful. Work through it in order.

### Step 1 — read the three-block pattern

Open fsm/seq101_moore.v. Identify the three blocks and, for each one, say what would break if you moved a line into a different block.

### Step 2 — run both styles against one stream

$ make fsm matches in the stream : 5 mismatches vs golden : 0 PASS - same language, Moore trails Mealy by one cycle, and the one-hot encoding is indistinguishable seq101_moore binary 13 cells 2 flip-flops seq101_moore one-hot 30 cells 4 flip-flops seq101_mealy binary 14 cells 2 flip-flops

### Step 3 — write one yourself

Draw the state diagram for a '110' detector with overlaps counted, on paper, before writing any code. Then write it in both styles, extend fsm/tb_seq101.v to drive it, and confirm the one-cycle offset holds for your machine too.

### Try these

▪  Re-encode the Moore machine as gray and measure the cells. Explain the result.

▪  Delete the default assignment from the next-state block, re-run the linter and then Yosys, and confirm the two agree about the latch.

▪  Add an illegal state to the Verilog machine by writing state <= 2'd7 in a debug branch. Then try the equivalent in the VHDL version and record what the analyser says.

Checkpoint I You can write a state machine in the three-block form without looking it up, choose Moore or Mealy for a stated reason, and predict what a change of encoding will cost.

## Tutorial J · A controller with a timer  (2 hours)

Open fsm/traffic.v. Note that the phase length lives in a down-counter, not in the state encoding — and that the state machine asks it exactly one question.

$ make fsm cycles checked : 40 property violations : 0 PASS - mutual exclusion and yellow-before-red both hold traffic controller 75 cells 10 flip-flops

### Step 1 — break it on purpose

Remove the yellow phases and re-run. Property P2 should fire, and the message should name the cycle. If it does not, your property is written wrongly — which is itself the lesson.

### Step 2 — extend it

Add a pedestrian request input that inserts an all-red phase. Then add a third property: after a pedestrian request, an all-red phase occurs within N cycles. Deciding what N should be is part of the exercise.

Checkpoint J You can write a property that fails when the design is wrong, and you can explain why a phase timer belongs in the datapath rather than in the state encoding.

## Tutorial K · Datapath and controller  (2 hours)

Open rtl/datapath_ctrl.v. Three modules: a controller, a datapath, and a parent that wires them together.

$ make dpctrl golden total : 157 hardware sum : 157 accum_ctrl (controller) 10 cells 2 flip-flops accum_datapath (datapath) 145 cells 24 flip-flops

### Step 1 — trace the control bundle

The run prints every control signal on every cycle. For each of the ten cycles, say which state the controller is in and why each signal has the value it has.

### Step 2 — move a decision, and see what it costs

Move the terminal-count test out of the datapath and into the controller, so the controller holds the counter itself. It will work. Then answer: what has the controller become dependent on that it was not before, and what would now have to change if the sample count went from 8 bits to 16?

### Try these

▪  Widen the datapath to 32-bit samples and confirm the controller needs no edit at all.

▪  Add a valid/ready handshake on the sample input. Which half changes?

Checkpoint K Given a block description you can say which signals are control, which are status and which are data, and justify each placement.

## Tutorial L · From a module to an IP  (2 hours)

$ make reuse delayline mismatches : 0 prescaler ratio errors : 0 delayline #(W=8, N) synthesised at four different depths: N = 1 8 cells 8 flip-flops N = 2 16 cells 16 flip-flops N = 4 32 cells 32 flip-flops N = 8 64 cells 64 flip-flops

### Try these

▪  Predict the cell count for N = 16 before running it.

▪  Add a bypass so that N = 0 removes the registers entirely and dout comes straight from din. This is the exercise worth arguing about: a generate loop with N = 0 builds nothing, so getting it right means understanding that the parameter is resolved before any hardware exists.

▪  Instantiate counter_n three times to divide by 4096, and check the ratio in simulation rather than assuming it.

▪  Write the three-line specification a colleague would need to use your delayline without asking you anything. If it needs a fourth line, the interface is wrong.

Checkpoint L You can parameterise a design so one file covers a family, and you can say precisely which language features survive into the netlist and which are elaborated away.

## Tutorial M · Two languages  (2 hours)

Open rtl/counter.v and vhdl/counter.vhd side by side and map every line of one onto the other. Then:

$ make langs IDENTICAL over all 18 cycles - including the wrap and the terminal count. IDENTICAL over all 17 cycles, detections included. (the '101' detector)

Then do the same with fsm/seq101_moore.v and vhdl/seq101_moore.vhd. The state machine is the more interesting comparison, because it is where the two languages genuinely differ rather than merely differing in spelling.

// Verilog: the states are NUMBERS you chose localparam [1:0] S_IDLE = 2'd0, S_1 = 2'd1, S_10 = 2'd2, S_101 = 2'd3; -- VHDL: the states are a TYPE, and nothing else can be assigned type state_t is (S_IDLE, S_1, S_10, S_101);

### Try these

▪  Change the width to 8 in both and re-run. Which file needed more editing?

▪  Introduce the same bug in both — remove the enable check — and confirm both fail in the same way.

▪  Write down three things VHDL made you say explicitly that Verilog let you leave implicit, and say which you prefer and why.

Checkpoint M You can read the VHDL counter without needing it translated, and name three concrete differences that are about notation rather than about hardware.

## Tutorial N · The flow, end to end, and the vendor tools  (2 hours)

$ make flow STAGE 1 SPECIFICATION 4 sentences, written before the RTL STAGE 2 LINT 0 issues STAGE 3 RTL SIMULATION 18 cycles; wraps at 15, tc correct STAGE 4 SYNTHESIS 12 cells STAGE 5 GATE SIMULATION the netlist, same stimulus STAGE 6 COMPARE IDENTICAL on all 18 cycles STAGE 7 PROVE Equivalence PROVEN by induction All seven stages passed.

### Step 1 — make each stage fail, one at a time

This is the exercise. Introduce a lint violation and watch stage 2 stop the flow. Break the counter's wrap and watch stage 3 catch it. Then find a change that passes stages 2 and 3 but fails stage 7 — that one takes thought, and finding it is the point.

### Step 2 — add a stage

Add a stage 8 of your own. A reasonable candidate: check that the netlist contains no latches, and fail if it does.

Checkpoint N1 You have seen every stage fail at least once, and you have added a stage that catches something the original seven did not.

### Step 3 — the same design in the vendor tools

Run the same counter through Vivado, and the same testbench through ModelSim. Compare three things with what you already have: the utilisation report against the Yosys cell count, the ModelSim transcript against the iverilog one, and the overall runtime.

Checkpoint N2 You have run one design through both toolchains and can account for the differences — and you have noticed which parts of the vendor flow correspond exactly to steps you already ran for free.

# Exercises

How to read the tags [H] hand work - do it on paper before you touch a keyboard. [C] computer - run it and record what you see. [W] write it down - a paragraph you could defend in a design review. Predict before you measure. Almost every [C] exercise here is worth more if you write down what you expect first. Running the command and copying the output teaches very little.

## Part A · What RTL means

6 exercises · 1 hour

A1  [H]  State the two things, and only the two things, that an RTL description says.

A2  [H]  In rtl/transfer.v, why is y equal to 1 rather than 6 on cycle 0?

A3  [H]  How many clock edges does the 5 take to reach acc, and why?

A4  [C]  Run make transfer. Add a fifth register after acc and predict the new latency before re-running.

A5  [W]  Explain, to someone who writes software, why the four assignments in that always block are not four steps.

A6  [W]  Why is RTL the level at which industrial design is done, rather than behavioural or gate level?

## Part B · The abstraction ladder

9 exercises · 2 hours

B1  [H]  Name the four levels of abstraction, highest first, and say in one phrase what you specify at each.

B2  [C]  Run make ladder. How many patterns were applied, and why is that number significant?

B3  [H]  Before running the synthesis step, predict which level will produce the fewest cells. Then check.

B4  [W]  Why did dataflow and gate produce the identical netlist?

B5  [H]  Count the transistors in nand2_sw. Why does a CMOS AND cost more than a CMOS NAND?

B6  [C]  Why did the switch-level description fail to synthesise?

B7  [W]  State the rule about choosing an abstraction level, and the reason for it.

B8  [W]  Name two things the tool is better at than you, and two things you are better at than the tool.

B9  [C]  Write a 4-bit adder at behavioural and gate level, synthesise both, and compare cell counts.

## Part C · Proof, not just testing

7 exercises · 2 hours

C1  [H]  What is a miter, and what question does the SAT solver answer?

C2  [C]  Run make prove. Why does the run end with a NOT EQUIVALENT line?

C3  [H]  fa_broken is missing (a & cin) from the carry. Which single input pattern does that make wrong?

C4  [C]  Remove a different term instead and predict the failing pattern before running the checker.

C5  [H]  A 32-bit adder has 65 inputs. How many patterns is exhaustive simulation, and how long at a million per second?

C6  [W]  Simulation and equivalence checking answer different questions. State each precisely.

C7  [C]  The counter's equivalence proof used induction rather than a single SAT call. Why?

## Part D · The synthesisable subset

10 exercises · 2 hours

D1  [C]  Run make subset. Which two constructs were refused, and in the tool's own words?

D2  [W]  Why can a synthesiser not build a while loop whose trip count depends on data?

D3  [H]  s07_forloop has a for loop and synthesised fine. What is the difference?

D4  [C]  a / b cost 371 cells and a / 4 cost 0. Explain both numbers.

D5  [C]  Change s10 to divide by 3. Predict the cell count first.

D6  [H]  What happened to the #5 delay in s05, and why is that dangerous?

D7  [W]  s06_initial synthesised successfully. Why is that a trap rather than good news?

D8  [C]  Write a construct you believe will be refused and check it.

D9  [W]  Give the two-minute procedure for deciding whether a construct is synthesisable.

D10  [H]  Which row in the table is the most dangerous, and why?

## Part E · Simulation against silicon

5 exercises · 1 hour

E1  [C]  Run make mismatch. How many of the six steps disagreed?

E2  [H]  Why did the other five steps agree?

E3  [C]  Change the list to always @(b) and predict which steps now disagree.

E4  [C]  Change it to always @* and confirm the disagreement count is zero.

E5  [W]  Explain in three sentences why this bug is worse than a syntax error.

## Part F · The coding rules

10 exercises · 2 hours

F1  [H]  State rules L001 and L002 and the reason for each.

F2  [C]  Run make lint on the good files and the bad files. How many rules fired, and on which lines?

F3  [C]  Run make lintcheck. What is it comparing, and why is that comparison worth making?

F4  [H]  Why does the linter count blocking and non-blocking assignments with two separate patterns rather than subtracting one from the other?

F5  [C]  Add a rule L008 of your own, with a file that breaks it and a file that does not.

F6  [C]  Find a file the linter gets wrong.

F7  [W]  Why can a regular expression not decide the latch question in general?

F8  [W]  Why lint before simulating rather than after?

F9  [H]  A colleague says the coding rules are style preferences. Respond.

F10  [W]  What does 'a fix you cannot explain is not a fix' mean here?

## Part G · Coding style

5 exercises · 1 hour

G1  [H]  Before running make mux, predict which of the three mux styles gives the fewest cells.

G2  [W]  The three were PROVED equivalent and still gave different cell counts. Is that a contradiction?

G3  [W]  Explain, from the source, why the case version cost more.

G4  [C]  Rewrite mux4_case as y = d[sel]; and measure. What happened?

G5  [W]  So which style should you write, and when does the answer change?

## Part H · The two pitfalls

7 exercises · 2 hours

H1  [H]  For the first six cycles of the stimulus, write q_nb and q_bl by hand before running make pitfalls.

H2  [C]  Run it. How many cycles was each version wrong on?

H3  [C]  Synthesise both. How many flip-flops did each build?

H4  [W]  Explain in two sentences why no tool warned about shift_bl.

H5  [C]  Synthesise s03_latch.v, confirm the $_DLATCH_ cell, then add an else and confirm it disappears.

H6  [H]  Write a combinational block with three outputs where exactly one is latched. Predict the netlist before synthesising.

H7  [W]  Both pitfalls are legal code that builds the wrong circuit. What does that tell you about relying on tool warnings?

## Part I · State machines

12 exercises · 3 hours

I1  [H]  Name the three blocks of the standard FSM pattern and say what each one is for.

I2  [W]  What would break if you moved the reset out of block 1?

I3  [H]  Why does the default assignment at the top of block 2 prevent a latch?

I4  [C]  Delete that default assignment, run the linter, then run Yosys. Do they agree?

I5  [H]  Draw the state diagram for a '110' detector with overlaps counted, before writing any code.

I6  [C]  Write it in both styles and confirm the one-cycle offset holds.

I7  [C]  Run make fsm. How many matches in the stream, and how many mismatches against the golden model?

I8  [W]  The golden model is computed from the stream rather than from either machine. Why does that matter?

I9  [H]  Which machine has more states, and which asserts its output sooner?

I10  [W]  When would you choose Moore even though it costs a cycle?

I11  [C]  Synthesise the binary and one-hot Moore machines. Which is bigger, and why?

I12  [C]  Re-encode the Moore machine as gray and measure. Explain the result.

## Part J · A controller with a timer

6 exercises · 2 hours

J1  [H]  The green phase lasts six cycles. Why is that not six states?

J2  [C]  Run make fsm. How many cycles were checked and how many properties were violated?

J3  [C]  Remove the yellow phases and re-run. What happens?

J4  [H]  Write the property that says the two roads are never both green.

J5  [W]  Why is a property better evidence than a waveform you inspected?

J6  [C]  Add a pedestrian request that inserts an all-red phase, and a third property to go with it.

## Part K · Datapath and controller

7 exercises · 2 hours

K1  [H]  Name the two halves of the accumulator and say what crosses between them.

K2  [C]  Run make dpctrl. For each of the ten cycles, say which state the controller is in and why each control signal has its value.

K3  [C]  What were the cell counts for the two halves?

K4  [C]  Widen the datapath to 32-bit samples. What had to change in the controller?

K5  [W]  Move the terminal-count test into the controller. It still works - so what did you lose?

K6  [H]  Classify each of start, done, sum, acc_en and cnt_done as control, status, data or interface.

K7  [W]  The latency is N + 3 clocks. Derive it rather than measuring it.

## Part L · From a module to an IP

6 exercises · 2 hours

L1  [H]  Name the three features that turn a module into reusable IP, and say when each one disappears.

L2  [C]  Run make reuse. What is the flip-flop count for each of N = 1, 2, 4 and 8?

L3  [H]  Predict the cell count for N = 16, then check it.

L4  [C]  Add a bypass so that N = 0 removes the registers entirely and dout comes straight from din.

L5  [C]  Instantiate counter_n three times to divide by 4096, and check the ratio in simulation.

L6  [W]  Write the three-line specification a colleague would need to use your delayline without asking you anything.

## Part M · Two languages

5 exercises · 2 hours

M1  [C]  Run make langs. What exactly was compared, and by what?

M2  [H]  Name three things VHDL made explicit that Verilog left implicit.

M3  [C]  Change the width to 8 in both files. Which needed more editing?

M4  [W]  Your colleague only knows VHDL. What can they read of your Verilog?

M5  [W]  Which language should someone starting today learn?

## Part N · The flow, and the vendor tools

8 exercises · 2 hours

N1  [C]  Run make flow. What evidence does each of the seven stages produce?

N2  [C]  Introduce a lint violation and confirm the flow stops at stage 2.

N3  [C]  Break the counter's wrap and see which stage catches it.

N4  [C]  Find a change that passes stages 2 and 3 but fails stage 7.

N5  [W]  Stage 6 and stage 7 both compare the RTL with the netlist. What is the difference?

N6  [C]  Add a stage 8 that fails if the netlist contains any latch.

N7  [W]  Why is 'a stage that produces no evidence' a problem?

N8  [W]  Which stage would you add next, and why?

# Worked Solutions

Every answer below was checked against the lab. Where a number is quoted it came from a real run; where a judgement is asked for, the solution gives the reasoning that earns the marks rather than a single word.

## Part A

A1  State the two things, and only the two things, that an RTL description says.

Which registers exist, and what transfers into each of them on each clock edge. Everything else - gate count, gate choice, wiring, speed - is left to synthesis.

A2  In rtl/transfer.v, why is y equal to 1 rather than 6 on cycle 0?

y <= x + 1 reads the OLD value of x, which was still 0 when the edge arrived. The 5 reaches x on that same edge, so it cannot reach y until the next one. Reading the block as a sequence rather than a table is what produces the wrong answer.

A3  How many clock edges does the 5 take to reach acc, and why?

Four. One per register in the chain x, y, z, acc - each transfer happens on its own edge, and nothing moves in between.

A4  Run make transfer. Add a fifth register after acc and predict the new latency before re-running.

Five edges. Each register added to a chain adds exactly one cycle of latency, which is the trade pipelining makes.

A5  Explain, to someone who writes software, why the four assignments in that always block are not four steps.

They are four wires into four registers, all loaded by the same clock edge. The semicolons separate declarations of what is connected to what, not instructions to be performed in turn.

A6  Why is RTL the level at which industrial design is done, rather than behavioural or gate level?

Above RTL you cannot say when things happen, so the tool has to invent a schedule; below it you cannot say anything else, so a human cannot write or read a real design. RTL is where a human can express intent AND a tool can build it without guessing.

## Part B

B1  Name the four levels of abstraction, highest first, and say in one phrase what you specify at each.

Behavioural - the function. Dataflow - the Boolean form. Gate - every gate and wire. Switch - individual transistors.

B2  Run make ladder. How many patterns were applied, and why is that number significant?

Eight, which is exhaustive: a full adder has three inputs, so 2^3 = 8 covers every possible case. There is nothing left untested.

B3  Before running the synthesis step, predict which level will produce the fewest cells. Then check.

Most people predict gate, reasoning that it is most specific. The answer is behavioural, with 5 cells against 6, because it left the tool free to choose the Boolean form and the tool found a better one.

B4  Why did dataflow and gate produce the identical netlist?

Writing the Boolean expression already commits to a structure. Naming the gates afterwards adds no information the tool did not already have, so it produces the same result from both.

B5  Count the transistors in nand2_sw. Why does a CMOS AND cost more than a CMOS NAND?

Four - two pmos in parallel, two nmos in series. An AND is a NAND followed by an inverter, so it costs six. This is why standard-cell libraries are full of NANDs and NORs.

B6  Why did the switch-level description fail to synthesise?

Transistor primitives have no gate-level meaning a synthesiser can map. They are what a library cell is BUILT from, not something you hand to a tool that builds from library cells.

B7  State the rule about choosing an abstraction level, and the reason for it.

Write at the highest level that expresses your intent, because every level you descend takes a decision away from the tool and gives it to you - whether or not you wanted it.

B8  Name two things the tool is better at than you, and two things you are better at than the tool.

Tool: choosing a gate mix, balancing a logic tree, meeting a constraint by restructuring, doing it consistently. You: how many cycles the job takes, where registers go, what is shared, what the interface looks like.

B9  Write a 4-bit adder at behavioural and gate level, synthesise both, and compare cell counts.

The behavioural version should be no larger and usually smaller. If yours is larger, look at whether your gate-level version accidentally shares logic the tool could not.

## Part C

C1  What is a miter, and what question does the SAT solver answer?

Both designs fed the same inputs, their outputs compared, and an assertion that the comparison always holds. The solver is asked to find an input pattern that breaks the assertion; failing to find one proves none exists.

C2  Run make prove. Why does the run end with a NOT EQUIVALENT line?

Because fa_broken.v is deliberately wrong. A checker that only ever reports success is not evidence of anything - you have to see it catch a real bug.

C3  fa_broken is missing (a & cin) from the carry. Which single input pattern does that make wrong?

a=1, b=0, cin=1. There the correct carry is 1 and the broken expression gives 0. Every other pattern is unaffected - one in eight.

C4  Remove a different term instead and predict the failing pattern before running the checker.

Removing (a & b) fails at a=1, b=1, cin=0. Removing (b & cin) fails at a=0, b=1, cin=1. In each case it is the pattern where that term is the only one asserting the carry.

C5  A 32-bit adder has 65 inputs. How many patterns is exhaustive simulation, and how long at a million per second?

2^65, about 3.7e19. At a million per second that is roughly 1.2 million years. This is the number that makes equivalence checking necessary rather than merely nice.

C6  Simulation and equivalence checking answer different questions. State each precisely.

Simulation: does this design produce the right output on the patterns I applied? Equivalence checking: do these two designs produce the same output on every possible input? Neither replaces the other - you still need simulation to know the SPEC is met.

C7  The counter's equivalence proof used induction rather than a single SAT call. Why?

The counter has state. Combinational equivalence compares outputs for one input vector; a sequential design needs an argument that holds across all reachable states, which is what induction over the state elements provides.

## Part D

D1  Run make subset. Which two constructs were refused, and in the tool's own words?

The data-dependent while loop ("While loops are only allowed in constant functions!") and real ("syntax error, unexpected TOK_REAL").

D2  Why can a synthesiser not build a while loop whose trip count depends on data?

It must produce a fixed amount of hardware. It cannot build 'however many iterations this value happens to need'. Doing that in hardware means a state machine taking several cycles - a design decision you must make explicitly.

D3  s07_forloop has a for loop and synthesised fine. What is the difference?

Its bounds are constants, so the tool unrolls it at compile time into seven parallel XOR gates. It is not a loop in hardware at all.

D4  a / b cost 371 cells and a / 4 cost 0. Explain both numbers.

Dividing by a signal requires a full combinational divider. Dividing by a constant power of two is a right shift, which is a renaming of wires and costs no logic whatsoever.

D5  Change s10 to divide by 3. Predict the cell count first.

Larger than 0 and much smaller than 371: division by a constant becomes a multiply-and-shift, which is real logic but far cheaper than a general divider.

D6  What happened to the #5 delay in s05, and why is that dangerous?

It was ignored; the module synthesised to one NOT gate. Dangerous because the simulation you signed off waited 5 ns and the silicon will not, so the two behave differently.

D7  s06_initial synthesised successfully. Why is that a trap rather than good news?

Yosys targets FPGAs, where the bitstream really does initialise flip-flops. An ASIC flow ignores initial blocks and the flops power up unknown. Code that works on an FPGA can therefore fail on an ASIC, with no warning at either end.

D8  Write a construct you believe will be refused and check it.

Candidates: file I/O ($fopen), hierarchical references across modules, unbounded recursion in a function, four-state logic used arithmetically. Record what the tool actually said, not what you expected.

D9  Give the two-minute procedure for deciding whether a construct is synthesisable.

Write a ten-line module containing only that construct, synthesise it, and read the output. Do not rely on memory or on advice about a different tool version.

D10  Which row in the table is the most dangerous, and why?

The incomplete sensitivity list. Every other row either fails loudly or produces something you can see in the cell count. That one produces a working netlist that does not match the simulation you tested.

## Part E

E1  Run make mismatch. How many of the six steps disagreed?

One. That is the whole problem: five of six steps agreed, so casual testing finds nothing.

E2  Why did the other five steps agree?

The RTL block only re-evaluates when a changes. On the steps where a changed, it recomputed and matched; on the steps where b changed while the result happened to be unaffected, both gave the same answer anyway.

E3  Change the list to always @(b) and predict which steps now disagree.

The mirror image: the block now sleeps through changes of a. Run it and confirm your prediction before reading the output.

E4  Change it to always @* and confirm the disagreement count is zero.

It is. @* builds the sensitivity list from what the block actually reads, and keeps it correct every time the block is edited.

E5  Explain in three sentences why this bug is worse than a syntax error.

A syntax error stops you immediately and costs a minute. This produces a design where the simulation and the silicon compute different things, so every test you write passes and the chip still fails. The bug is found in the lab, on hardware, months later.

## Part F

F1  State rules L001 and L002 and the reason for each.

L001: no blocking assignment in a clocked block, because two such blocks can see each other's half-updated values in an order the standard does not fix. L002: no non-blocking assignment in a combinational block, because it simulates like a register while synthesis builds logic.

F2  Run make lint on the good files and the bad files. How many rules fired, and on which lines?

The four good files are clean. s12_bad_style trips L001, L003, L004 and L006; s13 trips L007; s03 trips L005; s04 trips L004.

F3  Run make lintcheck. What is it comparing, and why is that comparison worth making?

The linter's L005/L006 latch prediction against whether Yosys actually put a $_DLATCH_ in the netlist. It turns two rules from opinions into predictions that have been checked on ten files.

F4  Why does the linter count blocking and non-blocking assignments with two separate patterns rather than subtracting one from the other?

The blocking pattern already excludes <=, >=, != and ==, so the two counts are disjoint. Subtracting would cancel real detections - which is a bug this tool had until the self-test caught it.

F5  Add a rule L008 of your own, with a file that breaks it and a file that does not.

Any defensible rule is acceptable. It must fire on the bad file, stay silent on the good one, and you must be able to say what bug it prevents.

F6  Find a file the linter gets wrong.

Not hard - it is regular expressions. Nested if/else across many lines, generate blocks, and unusual formatting will all fool it. The point is to know the limits of your own tool.

F7  Why can a regular expression not decide the latch question in general?

Deciding whether every path through a block assigns a signal requires understanding the block's control flow, which needs a parser and an analysis of reachability. Regexes match text, not structure.

F8  Why lint before simulating rather than after?

Linting costs seconds and catches a class of bug that simulation structurally cannot - the ones where the simulation itself is testing the wrong circuit.

F9  A colleague says the coding rules are style preferences. Respond.

Every one of the seven exists because breaking it produces a design that simulates differently from the way it is built. That is not taste; it is a correctness property, and the lintcheck target demonstrates two of them.

F10  What does 'a fix you cannot explain is not a fix' mean here?

Silencing a lint message by rearranging code until the tool stops complaining does not remove the hazard. If you cannot say which bug the rule prevents and why your change prevents it too, you have moved the problem rather than solved it.

## Part G

G1  Before running make mux, predict which of the three mux styles gives the fewest cells.

Most people predict all three are equal, on the grounds that the optimiser flattens style. The measured answer is 3, 6 and 10 - the conditional expression, the if/else chain and the case statement.

G2  The three were PROVED equivalent and still gave different cell counts. Is that a contradiction?

No. Equivalence is a statement about the function computed, and says nothing about area, timing or power. Equivalent is not identical.

G3  Explain, from the source, why the case version cost more.

The conditional expression hands sel[1] and sel[0] straight to the multiplexer selects. The case version builds equality comparators against 2'b00, 2'b01 and so on, and the tool then has to re-derive that those comparisons ARE the select bits. On Yosys 0.33 it gets part of the way.

G4  Rewrite mux4_case as y = d[sel]; and measure. What happened?

It should now match or beat mux4_assign, because a bit select IS a multiplexer and the tool no longer has to reconstruct one. Record what you actually got - the point of the exercise is the measurement, not the prediction.

G5  So which style should you write, and when does the answer change?

Whichever reads best to the next person, which for more than three branches is usually the case statement. The answer changes only when the block turns out to be on a critical path, and then you measure rather than guess.

## Part H

H1  For the first six cycles of the stimulus, write q_nb and q_bl by hand before running make pitfalls.

q_nb is a three-cycle delay of din. q_bl equals din replicated into all three bits, because each blocking assignment completes before the next reads it, so din races to q[2] within one cycle.

H2  Run it. How many cycles was each version wrong on?

Non-blocking: 0. Blocking: 6 of the 10 checked. The blocking version is only right by accident, on the cycles where din happened not to change.

H3  Synthesise both. How many flip-flops did each build?

Three and one. The blocking version did not build a slower or buggier shift register - it built a different circuit.

H4  Explain in two sentences why no tool warned about shift_bl.

Nothing illegal was written: blocking assignment in a clocked block is legal Verilog with well-defined semantics. The code simply describes a circuit other than the one the author had in mind, and no tool can know what was in the author's mind.

H5  Synthesise s03_latch.v, confirm the $_DLATCH_ cell, then add an else and confirm it disappears.

One latch before, none after. The same holds for s14_latch_case.v with a default added. Rules L005 and L006 predict both, and make lintcheck confirms the prediction against Yosys.

H6  Write a combinational block with three outputs where exactly one is latched. Predict the netlist before synthesising.

Two of the three outputs should be pure logic and the third a $_DLATCH_. The usual mistake is to assume the latch infects the whole block; it does not - inference is per signal, per path.

H7  Both pitfalls are legal code that builds the wrong circuit. What does that tell you about relying on tool warnings?

That warnings catch what the tool can prove is suspicious, which is a much smaller set than what is wrong. A methodology rule, checked mechanically, covers the rest - which is why the coding rules exist and why a linter runs before simulation.

## Part I

I1  Name the three blocks of the standard FSM pattern and say what each one is for.

Block 1, the state register: sequential, non-blocking, the only place a clock or reset appears. Block 2, next-state logic: combinational, a pure function of state and inputs. Block 3, output logic: combinational, reading the state (and the input too, if the machine is Mealy).

I2  What would break if you moved the reset out of block 1?

Nothing would work. Reset is by definition a sequential behaviour - it sets the value the register holds. Putting it in a combinational block would either do nothing or infer a latch.

I3  Why does the default assignment at the top of block 2 prevent a latch?

Because next_state is written unconditionally before any branch runs, so no path through the block leaves it unassigned, whatever the case statement covers or omits.

I4  Delete that default assignment, run the linter, then run Yosys. Do they agree?

They should: the linter raises L005 or L006 and Yosys builds a $_DLATCH_. make lintcheck automates exactly this comparison over sixteen designs.

I5  Draw the state diagram for a '110' detector with overlaps counted, before writing any code.

Four states for Moore: idle, seen 1, seen 11, seen 110 (accepting). From the accepting state a 1 goes to 'seen 1' and a 0 goes back to idle. Three states for Mealy, with the output on the arc out of 'seen 11' on a 0.

I6  Write it in both styles and confirm the one-cycle offset holds.

It must, for any machine: a Moore output is decoded from a register, so it cannot appear until the edge after the input that caused it.

I7  Run make fsm. How many matches in the stream, and how many mismatches against the golden model?

Five matches, zero mismatches - for the Moore machine, the Mealy machine and the one-hot encoding alike.

I8  The golden model is computed from the stream rather than from either machine. Why does that matter?

Because a model derived from the design under test passes whatever the design does. Computing it from the stimulus is what makes the test capable of failing.

I9  Which machine has more states, and which asserts its output sooner?

Moore has more states - it needs one whose only job is to say 'a match just finished'. Mealy asserts sooner, in the same cycle as the final input bit.

I10  When would you choose Moore even though it costs a cycle?

When the output leaves the block, drives other logic, or is timing critical. A Moore output is a decode of registered bits, so it is glitch-free and easy to constrain; a Mealy output inherits whatever the input does, glitches included.

I11  Synthesise the binary and one-hot Moore machines. Which is bigger, and why?

One-hot: 30 cells and 4 flip-flops against 13 and 2. Four states need four flip-flops instead of two, and the next-state logic drives four bits instead of two. One-hot pays off on an FPGA and as the state count rises, not here.

I12  Re-encode the Moore machine as gray and measure. Explain the result.

With four states gray is a permutation of binary, so expect the same two flip-flops and a cell count within a cell or two either way. Gray earns its keep when the state vector crosses a clock domain, not when it is decoded locally.

## Part J

J1  The green phase lasts six cycles. Why is that not six states?

Because the number of states would then depend on the timing, and a six-hundred-cycle phase would be impossible. The count belongs in a down-counter, and the state machine asks it one question: are you at zero?

J2  Run make fsm. How many cycles were checked and how many properties were violated?

Forty cycles, zero violations - and the properties are checked on every one of them, not sampled.

J3  Remove the yellow phases and re-run. What happens?

Property P2 fires and names the cycle. If it does not, the property was written wrongly - which is worth discovering now rather than on a design where you cannot check by hand.

J4  Write the property that says the two roads are never both green.

if (main_light == GREEN && side_light == GREEN) then flag an error. It is one line, it runs every cycle, and it keeps working after you change the design.

J5  Why is a property better evidence than a waveform you inspected?

Because it scales past the ten or so cycles a human can read, it runs again every time the design changes, and it states the requirement in a form a reviewer can check against the specification.

J6  Add a pedestrian request that inserts an all-red phase, and a third property to go with it.

A reasonable property: after a pedestrian request, an all-red phase begins within N cycles. Choosing N is part of the exercise, and it forces you to state a requirement the original specification left vague.

## Part K

K1  Name the two halves of the accumulator and say what crosses between them.

A datapath - accumulator and down-counter - and a controller of three states. Four control signals go down (acc_clr, acc_en, cnt_ld, cnt_dec) and one status signal comes up (cnt_done).

K2  Run make dpctrl. For each of the ten cycles, say which state the controller is in and why each control signal has its value.

Cycle 0 is S_IDLE with start high, so acc_clr and cnt_ld are asserted. Cycles 1-6 are S_RUN with cnt_done low, so acc_en and cnt_dec are asserted. Cycle 7 is S_RUN with cnt_done high, so nothing is asserted. Cycle 8 is S_DONE.

K3  What were the cell counts for the two halves?

10 cells and 2 flip-flops for the controller; 145 cells and 24 flip-flops for the datapath. Six per cent of the area holds all of the behaviour.

K4  Widen the datapath to 32-bit samples. What had to change in the controller?

Nothing. That is the property the split exists to give you, and it is worth verifying rather than assuming.

K5  Move the terminal-count test into the controller. It still works - so what did you lose?

The controller is now width-dependent: change the maximum sample count and you edit the state machine. Passing up a single status bit instead means the controller never changes at all.

K6  Classify each of start, done, sum, acc_en and cnt_done as control, status, data or interface.

start and done are the block's interface (a handshake). sum is data. acc_en is control, controller to datapath. cnt_done is status, datapath to controller.

K7  The latency is N + 3 clocks. Derive it rather than measuring it.

One cycle to accept the request, N to accumulate one sample each, one to notice the counter reached zero, one to flag it. The derivation holds for every N, which is why a testbench that only tries N = 6 has told you almost nothing.

## Part L

L1  Name the three features that turn a module into reusable IP, and say when each one disappears.

parameter and generate are elaborated away before synthesis; hierarchy survives until the tool flattens, if it flattens at all. None of the three exists in the final netlist.

L2  Run make reuse. What is the flip-flop count for each of N = 1, 2, 4 and 8?

8, 16, 32 and 64 - eight per stage, exactly N stages, linear. There is no loop anywhere in the netlist.

L3  Predict the cell count for N = 16, then check it.

128 cells and 128 flip-flops. If your prediction was different, look again at what the generate loop actually instantiates.

L4  Add a bypass so that N = 0 removes the registers entirely and dout comes straight from din.

A generate loop with N = 0 builds nothing, so tap[0] is already tap[N] and dout = din falls out naturally - but only if the array and the assignment are written so that N = 0 is legal. Getting this right means understanding that the parameter is resolved before any hardware exists.

L5  Instantiate counter_n three times to divide by 4096, and check the ratio in simulation.

Three four-bit counters cascaded on their terminal counts give 16 x 16 x 16 = 4096. Check it rather than assuming: the usual bug is enabling the second stage on the first stage's carry-out level rather than its one-cycle pulse.

L6  Write the three-line specification a colleague would need to use your delayline without asking you anything.

Width, depth, and the reset and enable semantics - what happens when en is low, and whether rst is synchronous. If it needs a fourth line, the interface is doing something surprising and should be changed rather than documented.

## Part M

M1  Run make langs. What exactly was compared, and by what?

The Verilog transcript from iverilog and the VHDL transcript from ghdl, compared line by line by diff. Identical over all 18 cycles.

M2  Name three things VHDL made explicit that Verilog left implicit.

Library and package declarations; the separation of entity from architecture; type conversions between unsigned and std_logic_vector. Verilog would have let you add 1 to anything.

M3  Change the width to 8 in both files. Which needed more editing?

Neither, if both were parameterised properly - which is the point of the exercise. If yours needed more editing in one language, that file was not as reusable as it looked.

M4  Your colleague only knows VHDL. What can they read of your Verilog?

Almost all of it: the clocked process, the reset, the enable, the vector, the wrap. What will need explaining is notation - blocking against non-blocking, what reg means, and case sensitivity.

M5  Which language should someone starting today learn?

The concepts first; the notation follows in an afternoon. In practice, whichever one the team uses - and expect to read the other. Large projects routinely mix them in one flow.

## Part N

N1  Run make flow. What evidence does each of the seven stages produce?

Spec: four sentences. Lint: 0 issues. RTL sim: 18 cycles with the wrap checked. Synthesis: 12 cells. Gate sim: 18 cycles. Compare: identical. Prove: equivalence by induction.

N2  Introduce a lint violation and confirm the flow stops at stage 2.

It should. A methodology in which a failing stage does not stop the flow is a checklist people tick, not a set of gates.

N3  Break the counter's wrap and see which stage catches it.

Stage 3, the RTL simulation, because the spec said it must wrap and the transcript is checked against that.

N4  Find a change that passes stages 2 and 3 but fails stage 7.

Anything that is functionally correct on the stimulus applied but not equivalent in general - for example a reset that is asynchronous in the RTL and synchronous in the netlist, or a wrap condition that differs only in a state the 18-cycle test never reaches.

N5  Stage 6 and stage 7 both compare the RTL with the netlist. What is the difference?

Stage 6 compares them on the 18 cycles that were tested. Stage 7 proves they agree on every input sequence, by induction, without enumerating any.

N6  Add a stage 8 that fails if the netlist contains any latch.

Grep the JSON for DLATCH, or count $_DLATCH_ cells and exit non-zero if the count is not zero. This is a real check that real teams run.

N7  Why is 'a stage that produces no evidence' a problem?

Because nobody can tell whether it was run. A methodology is auditable only if each stage leaves something behind that a reviewer can look at.

N8  Which stage would you add next, and why?

Reasonable answers: code coverage on the RTL simulation; a check that every output is driven; a constraint check; a second synthesis with different options to detect fragility. Any answer that names the bug the stage would catch.

# Reference Card

## What RTL is

An RTL description states TWO things and nothing else: 1. which registers exist 2. what transfers into each one, on each clock edge Everything between the registers is combinational logic with one clock period to settle. You never say how long it takes or which gates it uses.

## The two assignment operators

<= NON-BLOCKING in CLOCKED blocks read every right-hand side first (old values), then update all left-hand sides at the same instant. a<=b; b<=a; SWAPS them. = BLOCKING in COMBINATIONAL blocks each statement completes before the next starts, like software. a=b; b=a; does NOT swap.

## The four levels of abstraction

| Level | You specify | Synthesises? | Cells (full adder) |
| --- | --- | --- | --- |
| behavioural | the function | yes | 5 |
| dataflow | the Boolean form | yes | 6 |
| gate | every gate and wire | yes | 6 (identical netlist) |
| switch | individual transistors | no | refused |

Write at the highest level that expresses your intent. Every level you descend takes a decision away from the tool and gives it to you.

## The three-block state machine

always @(posedge clk or negedge rst_n) // BLOCK 1 state register if (!rst_n) state <= S_IDLE; // sequential, <= else state <= next_state; // reset lives HERE only always @(*) begin // BLOCK 2 next-state logic next_state = state; // THE DEFAULT ASSIGNMENT case (state) ... endcase // combinational, = end always @(*) begin // BLOCK 3 output logic out = 1'b0; // defaults again case (state) ... endcase // state only => MOORE end // state + input => MEALY

## Moore and Mealy

|  | Moore | Mealy |
| --- | --- | --- |
| output depends on | state only | state and input |
| output appears | one cycle after the input | the same cycle |
| output glitches | no | yes - inherits the input's |
| measured here | 13 cells, 2 flops | 14 cells, 2 flops |

## Datapath and controller

CONTROLLER a state machine. Narrow. All the decisions. DATAPATH registers, adders, muxes. Wide. No decisions. control signals go DOWN acc_clr acc_en cnt_ld cnt_dec status signals come UP cnt_done measured on the accumulator: controller 10 cells, datapath 145 cells.

## The seven coding rules

| Rule | What it catches |
| --- | --- |
| L001 | blocking (=) in a clocked block - use <= |
| L002 | non-blocking (<=) in a combinational block - use = |
| L003 | = and <= mixed in one always block |
| L004 | explicit sensitivity list - use always @* |
| L005 | if with no else in a combinational block - infers a latch |
| L006 | case with no default - infers a latch |
| L007 | signal driven from more than one always block |

## The synthesisable subset, at a glance

ALWAYS FINE always @* · always @(posedge clk) · assign for loops with constant bounds · case with default parameters and localparams · module instantiation NEVER # delays · real · data-dependent while loops file I/O · most system tasks TRAPS initial accepted by FPGA tools, ignored by ASIC flows if/case with no else/default silently builds a LATCH always @(a) synthesis ignores the list; simulation does not a / b 371 cells. a / 4 is 0 cells.

## Commands

make transfer what register transfer level literally means make ladder one adder at four levels, all simulated together make prove formal proof that those levels are the same circuit make mux one function, three styles: proved, then measured make subset which constructs synthesise, and what they cost make mismatch RTL and its own netlist, disagreeing make pitfalls the inferred latch and the blocking-assignment trap make lint the seven rules, checked make lintcheck and cross-checked against what Yosys builds make fsm Moore, Mealy, encoding, and a controller with a timer make dpctrl datapath + controller, every control signal per cycle make reuse parameters, hierarchy and generate, measured at 4 depths make langs counter and FSM, in Verilog and VHDL, both run make flow the whole methodology: spec to formal proof make all of the above, in order python3 tools/rtl_lint.py <file.v> ... ./scripts/equiv.sh fileA.v topA fileB.v topB ./scripts/stat.sh <label> <top> <file.v> cells and flip-flops

The three sentences to leave with 1. RTL says which registers exist and what transfers into them. Everything else is the tool's problem. 2. Write at the highest level that expresses your intent - and then check what the tool did with it. 3. The dangerous bugs are the ones where the simulation and the silicon are not the same circuit. Every coding rule in this topic exists to prevent one of them. 4. Almost every block is a datapath and a controller, and almost every controller is a state machine written in three blocks. Learn those two shapes and most designs stop looking unfamiliar.